SensorWeb Sun SPOT App Suite

Peng Deng (dengpeng.cn@gmail.com)

This folder contains Sun SPOT source code for SensorWeb. Current SunSPOT SDK is version Blue(4.0).

There are three NetBeans projects.

- SensorWeb_SunSPOT_SensorSide: is the embedded app source code which should deploy to and running on Sun SPOTs.

- SensorWeb_SunSPOT_SerialForwarder: is the host app source code running on PC which has a Sun SPOT base station connected to.

- SensorWeb_SunSPOT_SerialForwarder_Tester: is a tester app to test things work fine.

Quick Start Guide

- Install NetBeans and J2SE

The current running system using NetBeans 6.5 and J2SE 1.6.11

- Install SPOTManager from http://www.sunspotworld.com/SPOTManager/

It will automatically install ANT and NetBeans plugin

- Please use SunSPOT SDK Version Blue (4.0), and ensure your SunSPOTs and SunSPOT basestation have the version Blue firmware running

To check you have right versionof SDK on your PC, run SPOTManager. You should see the current active SDK is version blue-080827

To check SunSPOTs and basestation have the right firmware version, please connect SunSPOT to USB port. Then, in the SPOTManager, under the “Sun SPOTs” tab, click SPOT info button and see the printout. If the printout says “[java]    spot.sdk.version: blue-080827”, you have the right configuration otherwise you need to upgrade or downgrade your firmware. For more information please refer to http://www.sunspotworld.com/

- Build and deploy “SensorWeb_SunSPOT_SensorSide” to SunSPOTs

Open “SensorWeb_SunSPOT_SensorSide” from NetBeans. Right click on the opened project and select “Build project + Deploy to Sun SPOT”, if you did not see this option, you should install NetBeans SunSPOT plugin from https://netbeans-spot.dev.java.net/

- Connect SunSPOT basestation to host PC and run “SensorWeb_SunSPOT_SerialForwarder”

Open “SensorWeb_SunSPOT_SerialForwarder” from NetBeans. Right click on the opened project and select “Run”, if you did not see this option, you should install NetBeans SunSPOT plugin from https://netbeans-spot.dev.java.net/

- Run “SensorWeb_SunSPOT_SerialForwarder_Tester”

Open “SensorWeb_SunSPOT_SerialForwarder” from NetBeans and run it.

If you can see similar output below, your SunSPOT SerialForwarder is running properly

- Common errors

  - You may need to change the entries of required Java libraries according to your SunSPOT SDK in project properties

1. SensorWeb_SunSPOT_SensorSide

Input: DURATION;FREQUENCY;SENSINGTYPE

Input example: 6000;1000;light

Output: SENSINGTYPE;VALUE;TIMESTAMP

Output example: light;120;123456

Radiogram socket listens on port 100

Radiogram socket sends output to port 100

2. SensorWeb_SunSPOT_SerialForwarder:

Input: DURATION;FREQUENCY;SENSINGTYPE;MAXHOP

Input example: 6000;1000;light;3

Output: SENSINGTYPE;VALUE;TIMESTAMP

Output example: light;120;123456

Radiogram socket listens on port 100

Radiogram socket sends output to port 100

TCP socket listens on port 9004

TCP socket sends packet to port 9007

3. SensorWeb_SunSPOT_SerialForwarder_Tester

Output: DURATION;FREQUENCY;SENSINGTYPE;MAXHOP

Output example: 6000;1000;light;3

TCP socket listens on port 9007

TCP socket sends packet to port 9004